Patient-Specific Finite Element Modeling of Scoliotic Curve Progression using Region-Specific Stress-Modulated Growth

Christian R. D’Andrea, Amer F. Samdani, Sriram Balasubramanian

Include Affiliations and ORCIDs

Confirm

Fill

Check

[Ref]  normative paper reference

# Abstract

Purpose:  This study describes the creation of patient-specific osteo-ligamentous finite element (FE) models of the spine, ribcage, and pelvis, simulation of up to three years of region-specific, stress-modulated growth, and validation of simulated curve progression with patient clinical angle measurements. Research Question: does the inclusion of region-specific, stress-modulated vertebral growth, in addition to scaling based on age, sex, weight, skeletal maturity, and spine flexibility allow for clinically accurate scoliotic curve progression prediction in patient-specific FE models of the spine, ribcage, and pelvis?

Methods: Frontal, lateral, and lateral bending X-Rays of five AIS patients were obtained for approximate three-year timespans. Patient-specific FE models were generated by morphing a normative template FE model with landmark points obtained from patient X-rays at the initial imaged timepoint. Vertebral growth behavior and response to stress, as well as model material properties were made patient-specific based on several prognostic factors. Model angle outputs were compared to the corresponding X-ray measurements.

Results: Average FE model errors were approximately 6°, 12°, 9°, and 5° for thoracic Cobb, lumbar Cobb, kyphosis, and lordosis angles, respectively. Average error in prediction of vertebral wedging in the apex and adjacent levels was approximately 3°. Average vertebral column stress ranged from 0.11 MPa in tension to 0.79 MPa in compression.

Conclusion: Integration of region-specific stress-modulated growth, as well adjustment of growth and material properties based on patient-specific data yielded clinically useful prediction accuracy while maintaining physiological stress magnitudes. This framework can be further developed for surgical simulation.

Keywords: Adolescent Idiopathic Scoliosis, Progression, Hueter-Volkmann, Growth Modulation, Skeletal Maturity, Finite Element Model

# Introduction

Adolescent idiopathic scoliosis (AIS) is a complex three-dimensional (3D) deformity of the spine defined by a progressive frontal plane curvature and axial rotation affecting 1-3% of 10-16 year-olds in the U.S. [1]. This condition has been associated with a perception of physical limitation and a decrease in self-esteem and body image [2]. The formation of lateral spine curvature observed in AIS is associated with alterations in the stress profile across vertebral epiphyseal growth plates, which has been shown to alter local growth rates [3]. The Hueter-Volkmann law, the guiding principle of growth modulation in AIS spine, states that growth is stimulated in relative tension and inhibited in relative compression [4]. This law was validated across multiple species and anatomical locations, and has been shown to produce predictable alterations in bone growth when a known loading regimen is applied [5]. Quantifying the relationship between applied stress and resulting growth in the human pediatric scoliotic spine to predict curve progression on a patient-specific basis can assist in the optimization of the type and timing of intervention.

Finite Element (FE) methods using beam elements have been widely used to model asymmetric stress to simulate progressive AIS [6-12]. More recently, FE models consisting of both volumetric (tetrahedral or hexahedral) and beam elements have been reported, allowing for more detailed analysis of stress applied to the vertebral epiphyses [12, 13]. Compressive stresses in these volumetric models, based on gravity and muscle stabilization forces, determine growth modulation of vertebral body height [13]. While these models were validated with longitudinal clinical data from patients with progressive scoliosis, they are limited by simplifications of vertebral growth that do not consider direction- and region-specific variations in growth rates for all anatomical regions of the vertebrae, including the posterior regions, through which 3-25% of longitudinal-axis compressive stress may be transferred [14, 15]. Recently published comprehensive data on region-specific normative pediatric vertebral morphology and size-invariant shape [16-18] have been used to validate region-specific orthotropic growth in a normative 10-year-old T1-L5 spine [15]. Such data could be used to incorporate asymmetric stress-modulated orthotropic region-specific growth in scoliotic spine FE models.

Maximizing the accuracy of material property assignment also plays a crucial role in simulation of stress-modulated growth; parameters such as intervertebral disc (IVD) elastic modulus and ligament stiffness may alter stress distribution at the vertebral epiphyses. While linear elastic material properties for vertebrae, intervertebral discs, and ligaments, have been implemented in several prior models, a recent study from our lab used age- and level-specific non-linear mechanical properties for the intervertebral ligaments to improve model biofidelity [15, 19-21]. Additionally, since pediatric tissue properties are not widely available, age-based scaling methods have been applied to level-specific properties obtained from adult cadaveric specimen [22, 23]. However, no study has implemented age- and level-specific material properties in an osteo-ligamentous pediatric scoliotic spine FE model which incorporates region-specific, stress-modulated growth.

A longitudinal study from our lab illustrated significant differences in vertebral growth patterns between normative and scoliotic vertebrae in skeletally immature rabbits [24]. While comprehensive reporting of normative vertebral shape and morphology with growth has been reported, similar data aren’t available for the scoliotic vertebral growth in humans [16, 18, 25]. Additionally, variations (biological and/or inter-subject) in several prognostic factors including age, sex, weight, skeletal maturity, and spine flexibility, contribute to variable scoliotic curve progression [3, 26-28][Ausilah’s RF Paper]. These prognostic factors have not yet been integrated together in an FE modeling framework of AIS with region-specific, stress-modulated vertebral growth; therefore, this integration may improve patient-specific prediction of curve progression [13, 29].

The objective of the current study is to simulate and validate region-specific stress-modulated vertebral growth in patient-specific scoliotic spine FE models which integrate age, sex, weight, skeletal maturity, and spine flexibility. Such FE models can serve as a tool to predict AIS progression, and can also aid in decision making for clinical intervention. The current study will build upon our previously published work on pediatric patient-specific FE modeling and growth [15, 30].

# Methods

## AIS Patient Selection

After institutional review board approval, frontal and lateral low-dose biplanar radiographs (EOS Imaging Inc, Paris) of five AIS patients were obtained from the Shriners Hospitals for Children, Philadelphia. These patients were either unbraced (three patients), or non-compliant with brace wear (two patients), such that bracing effects were considered negligible. Table 1 shows a patient cohort that ranges in age, sex, Lenke type, Risser sign, initial and final Cobb angles, and spine flexibility ratio (defined in Equation 1).

Table 1: AIS Patients for whom patient-specific FE models were created. LB = Lateral Bending

| Patient Number | Initial Age (yrs), Sex (M/F) | Lenke Type | Initial Risser Sign | Cobb Angle (°) [Initial, Final] | Cobb Angle during LB into Convexity (°) | Spine Flexibility Ratio (SFR) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 11F | 1AN | 0 | 38.6, 46.3 | 13.7 | 0.70 |
| 2 | 11F | 1A- | 1 | 42.5, 50.1 | 18.5 | 0.63 |
| 3 | 16M | 5CN | 2 | 48.9, 52.9 | 23.2 | 0.56 |
| 4 | 13M | 1AN | 3 | 39.5, 50.8 | 26.2 | 0.48 |
| 5 | 14F | 1CN | 4 | 48.9, 52.9 | 36.8 | 0.30 |

## Vertebral Geometry Reconstruction from Bi-planar Radiographs and Finite Element Mesh Generation

A total of 153 vertebral landmark points (nine per vertebra) were selected in the frontal and sagittal radiographs and registered by triangulation using a custom code (MATLAB 2020b, MathWorks, Natick, MA) [15, 31]. 3D reconstructions of vertebrae using the landmark points were validated by comparing 3D landmark point locations obtained from frontal and lateral radiographs to those obtained directly from a chest CT scan. The average reconstruction surface deviation was 3.0 ± 2.2 mm. Prior subject-specific reconstruction methods reported similar accuracies [30, 32].

For all subjects, after landmark point registration, subject-specific FE models were generated using previously a reported dual-kriging method [30, 33, 34], which morphs a normative FE spine model template to subject-specific spine geometry based on vertebral landmark points. The normative FE template utilized was a hexahedral FE model of a 10-year-old osteo-ligamentous thoracic and lumbar spine (T1-L5 with IVDs), ribcage, and pelvis with age- and level-specific ligament properties and orthotropic region-specific vertebral growth [15]. After morphing the FE template model to each subject-specific geometry, mesh quality was compared to previously established acceptability criteria metrics [30, 35].

Figure 1 (left): The template normative osteo-ligamentous FE model utilized to generate patient-specific models, which includes spine, IVD, ribcage, pelvis, and intervertebral ligaments.

Figure 2 (right): A) Example (Patient 3) Frontal X-Ray, B) Patient-specific landmark point selections (blue=frontal vertebral body corners, red=sagittal vertebral body corners), normative FE template model, C) Patient-specific FE model. In B and C, Ribcage, pelvis, and ligaments are hidden.

## Assignment of Subject Age- and Level-Specific Material Properties

All anatomical structures aside from the ligaments (cortico-cancellous bone of the vertebrae, ribs, sternum, and pelvis, IVD, and costal, costo-vertebral, and transverse joint cartilage) were modeled as linear elastic materials, while all vertebral ligaments were modeled as tension-only springs. Age- and level-specific material properties were assigned to the anatomical structures in each subject-specific FE model, shown in Table 1. The values for each mechanical property were obtained from published adult values which were adjusted based on chronological age-based scale factors [22, 36-40]. A comprehensive table of level-specific material properties of spinal ligaments are reported in the Appendix of previous work from our lab [15]. Furthermore, to increase patient-specificity of the FE model, IVD elastic modulus was scaled linearly with patient spine flexibility ratio (Equation 1), as shown in Equation 2:

SFR=CAS-CALBCAS 		   			   (1)

where, SFR = Spine Flexibility Ratio, CAS = Cobb Angle while Standing (at Final Timepoint), and CALB  = Cobb Angle during Lateral Bending into Convexity (at Final Timepoint).

And the patient-specific, scaled elastic modulus of IVD, Escaled, was found with Equation 2:

Escaled= Ebaseline1-C*Flexibility	    		         (2)

Where C is a constant, solved with iterative error minimization to be 0.3 [28].

Table 2: Age- and level-specific material property assignments

| Anatomical Structures | Adult Elastic Modulus (E, MPa) or Stiffness (k, N/mm) [Reference(s)] | Example Scale Factor, Patient 3 [22] | Example Scaled Elastic Modulus (E, MPa) or Stiffness (k, N/mm), Patient 3 [Reference(s)] |
| --- | --- | --- | --- |
| Vertebrae | E = 350 [22, 36] | 0.95 | E = 332.5 |
| IVDs | E = 20 [41] | 0.95 | E = 16.8 |
| Ligaments | Level-specific ligament properties [15] | 0.97 | Age-and Level-specific Force-deflection curves [15] |
| Ribs | E = 2100 [39] | 0.95 | E = 1995.0 |
| sternum | E = 2100 [39] | 0.95 | E = 1995.0 |
| Costal cartilage | E = 10.4 [37, 38] | 0.95 | E = 9.9 |
| Costo-vertebral & transverse joint cartilage | k = 48.9 [40] | 0.94 | k = 45.9 |
| Pelvis | - | - | E= 5000 [41] |

## Asymmetric Stress-Modulated Growth Implementation

Vertebral growth was modeled through adaptation of a region-specific orthotropic thermal expansion method described by Balasubramanian et al. (2021) [15]. This method allocated 663 thermal expansion coefficients (for 17 vertebral levels x 13 regions x 3 directions), which were initially assigned values corresponding to normative age- and sex-based vertebral growth strains. To represent the stress-modulated asymmetric growth that is observed in AIS subjects, normative thermal expansion coefficients for vertical growth in vertebral bodies were scaled using compressive stress in the corresponding IVD region (i.e. anterior left vertebral body growth is scaled based on compressive stress in anterior left IVD region). Each growth rate was updated once per time interval documented for each patient. To achieve this scaling, a linear relationship between stress and growth rate was utilized, defined by Stokes et al. (2006) [3]. In Equation 3, G is resultant growth (mm), Gm is baseline growth under normative stress (mm), β is stress sensitivity (MPa-1), σ is the altered stress (MPa), and σm is the baseline stress (MPa). The value for β was initially defined as 0.4 MPa-1 as reported by Shi et al. (2009), and adjusted linearly according to Risser sign such that as a patient approached Risser 5, β would approach zero [42].

G= Gm[1+βσ-σm] 					(3)

## Boundary Conditions (Contacts and Constraints), and Loading Conditions (Gravitational Force)

The pelvis was constrained in all translation and rotation directions, while the T1 vertebra was limited only to vertical translation. The articulating surfaces between facets were defined as sliding contacts, while the interfaces between IVDs and vertebral endplates were defined as tied contacts. Gravity was applied through an adaptation of a method by Clin et al. (2011), wherein global vertical ‘anti-gravity’ forces were applied in the upward direction, all stresses were reset, and gravity forces of the same magnitudes were then applied in the downward direction, pairing spine positioning (standing) with a corresponding stress profile [26]. The magnitude of each force vector was determined based on percentage of bodyweight acting at each level, and applied from the geometric centroid of each vertebra. After gravity application, the total principal compressive stresses in each region were computed.

## Validation of Spine Curvatures and Vertebral Wedging with Growth

Each patient-specific FE model was validated with measures of thoracic and lumbar Cobb angles, kyphosis, lordosis, axial rotation, and wedging of the apical vertebral body, at the time of reconstruction and at each subsequent timepoint. A custom MATLAB code was created to extract these measurements at each reported timepoint, manually from each patient radiograph and automatedly from each patient-specific FE model. Error thresholds between radiograph-obtained measurements and those extracted from each FE model were set at 5% strain error for length measurements, and 5° for all angles, based on human error involved in manual angle extraction from radiographs [43].

# Results

Patient-specific osteo-ligamentous FE models with ribcage and pelvis, with age- and level- specific material properties were generated for five AIS patients, stress-modulated growth was simulated at each radiograph acquisition timepoint, and FE simulation results were compared to data extracted from patient radiographs at each corresponding timepoint. For all FE models generated, mesh quality met the following criteria: Jacobian ≥ 0.5 (99.3%), aspect ratio ≤ 5 (99.7%), skewness ≤ 60° (99.2%), warpage ≤ 40° (99.6%), quadrilateral face minimum angle ≥ 30° (98.7%), and quadrilateral face maximum angle ≤ 150° (97.8%) [30, 35].

## Spine Curvatures

Average errors in thoracic and lumbar Cobb angles were 6.3±4.6° and 12.2±6.6°, respectively, and average errors in kyphosis and lordosis angles were 8.9±7.7° and 5.3±3.4°, respectively (Figure 3, Table 3).

Figure 3: Comparison of clinical indices on patient frontal (first row) and sagittal (second row) x-rays and FE models (third row). Red models indicate initial timepoint, and blue indicate model solution after three years of simulated growth. In the third row, left models in each column are frontal views and right models in each column are sagittal views, and top degree values indicate measurements at zero years, while bottom degree values indicate measurements after three years of simulated growth. Ribs and ligaments have been removed for improved spine visibility.

Table 3: Error between the current FE models and patient clinical indices. ITP = initial time point.

|  | ITP | ITP+1y | ITP+2y | ITP+3y | All TPs |
| --- | --- | --- | --- | --- | --- |
| Thoracic Cobb (°) | 5.7 ± 4.4 | 5.5 ± 4.6 | 7.4 ± 5.5 | 6.2 ± 5.2 | 6.3±4.6 |
| Lumbar Cobb (°) | 10.9 ± 5.2 | 11.4 ± 7.7 | 13.0 ± 6.9 | 13.5 ± 7.2 | 12.2±6.6 |
| Kyphosis (°) | 8.8 ± 6.6 | 9.6 ± 7.6 | 7.7 ± 6.4 | 12.5 ± 11.3 | 8.9±7.7 |
| Lordosis (°) | 8.2 ± 7.4 | 5.5 ± 3.0 | 7.0 ± 6.3 | 3.1 ± 1.8 | 5.3±3.4 |
| Axial Rotation (°) | 6.7 ± 6.6 | 7.6 ± 9.4 | 10.1 ± 10.9 | 12.3 ± 13.1 | 10.8 ± 11.1 |

## Vertebral Wedging

Wedging angle between the superior and inferior faces of the apical and two adjacent vertebral bodies was predicted to within an average error of 3.2±2.2° (Table 4). Furthermore, error in change in wedging angle per year was 0.99±0.95°, difference between FE models and patient X-Rays shown in Figure 4.

|  | ITP | ITP+1y | ITP+2y | ITP+3y | All TPs |
| --- | --- | --- | --- | --- | --- |
| Apex +1 (°) | 2.9 ± 2.1 | 2.9 ± 2.4 | 3.0 ± 2.3 | 3.9 ± 3.6 | 3.2 ± 2.7 |
| Apex (°) | 4.1 ± 1.7 | 2.9 ± 1.5 | 3.1 ± 1.5 | 3.4 ± 2.0 | 3.4 ± 1.8 |
| Apex -1 (°) | 2.3 ± 2.1 | 3.0 ± 1.8 | 3.5 ± 1.7 | 2.4 ± 1.7 | 2.8 ± 1.9 |

Figure 4: Comparison of change in wedging angle per year for the apical and adjacent levels, between FE models and patient X-rays.

## Stress in IVDs

Average principal stress in each IVD region for the apical vertebral level (IVD below apical vertebra) is shown in Table 5 for patient-specific FE model.

|  | ITP | ITP+1y | ITP+2y | ITP+3y | Δ |
| --- | --- | --- | --- | --- | --- |
| Anterior, Convex (MPa) | 0.34 ± 0.10 | 0.06 ± 0.14 | 0.04 ± 0.16 | 0.00 ± 0.17 | -0.34 ± 0.07 |
| Anterior, Concave (MPa) | 0.34 ± 0.10 | 0.66 ± 0.10 | 0.74 ± 0.09 | 0.79 ± 0.11 | 0.46 ± 0.01 |
| Posterior Convex, (MPa) | 0.20 ± 0.10 | -0.07 ± 0.05 | -0.09 ± 0.06 | -0.11 ± 0.07 | -0.31 ± 0.03 |
| Posterior, Concave (MPa) | 0.20 ± 0.10 | 0.49 ± 0.24 | 0.73 ± 0.30 | 0.73 ± 0.32 | 0.54 ± 0.21 |

# Discussion

This is the first study to implement orthotropic region-specific stress-modulated growth in patient-specific FE models of the osteo-ligamentous T1-L5 spine, ribcage, and pelvis, with vertebra, ribcage and pelvis comprised entirely of volumetric elements, and with age- and level- specific material properties.

The errors in clinical indices measured during and after three years of simulated growth were under 5°, 10°, and 15° error thresholds in 38%, 72%, and 83% of angular measurements, respectively, across all patients and timepoints. For thoracic Cobb angle in particular, these percentages were 47%, 84%, and 90%, respectively. Curve progression varied with age, sex, Lenke type, Risser sign, initial and final Cobb angles, and flexibility, and yearly progression averaged 3.2±5.4°/year. The observed FE model Cobb angle progression rates were considered realistic based on yearly curve progression rates observed in both the patient radiographs accessed for the current study, and on report of median AIS curve progression rate being approximately 7°/year [44]. Previous reports of apical vertebral wedging in silico from 0.6 to 1.4°/year corroborate the results of the current model, the corresponding average values in the current study ranged from 0.5 to 0.8°/year at the apical and immediately adjacent levels [29]. Furthermore, average vertebral column stress ranged from 0.11 MPa in tension to 0.79 MPa in compression, similar to the stress range of 0.3 MPa in tension to 0.7 MPa in compression reported by Clin et al. (2011) [26]. Together, these results suggest that the current modeling approach can predict alterations in spine geometry and related vertebral wedging within the patient cohort with clinically useful accuracy.

*Limitations*

The current FE model is limited by being solely osteo-ligamentous – the lungs, muscle, and any connective tissue beyond the inter-vertebral ligaments were not included. These features, if included in future analysis, may alter the stress environment of the spine as a whole, and therefore alter the calculated stress sensitivity that would accurately represent curve progression. However, the stress sensitivity parameters calculated in the current FE model may be considered to compensate for its limited scope. Second, due to the relative coarseness of the Risser sign (0-5, in steps of 1) to chronological age (continuous), scaling of stress sensitivity based on a correlation between Risser sign and chronological age may contribute to error in prediction of curve progression. This limitation could be addressed by correlating a more precise (Sanders score) or even continuous scale (Collagen X biomarker levels) of skeletal maturity with chronological age, and using this relationship to more accurately scale stress sensitivity for each patient [45, 46]. Third, IVD elastic modulus was scaled from normative data for all simulated timepoints based on flexibility obtained at the final timepoint (approximately 3 years after ITP), where in reality, IVD elastic modulus may vary between normative and AIS patients, and flexibility may not remain constant over this timespan [47]. Future studies may define IVD material property differences between normative and AIS patients. Additionally, IVD elastic modulus may be updated at each timepoint according to flexibility ratio by obtaining lateral bending imaging at those respective timepoints. Fourth, the effect of activity levels resulting from various exercise regimens on both reducing curve progression or decreasing an existing Cobb angle is considered significant based on a review of 19 publications [48]. However, the activity levels of the patients included in this study are unknown.

Stress effects on growth introduced by bracing were not accounted for, though brace wear compliance was reported to be low in the clinical notes and therefore considered negligible [49]. Further improvements to the modeling framework can consider vertebral stresses induced by externally applied loads from bracing. No patient activity level data was obtained, which may act as a source of prediction error. Inclusion of such data may prove to be a significant feature in precise and accurate curve progression. Fourth, while sex-specific normative baseline growth rates have been established, no sex-specific stress sensitivity or rate-of-change of stress sensitivity has been established, despite prevalence of AIS in females being higher [50]. While the effect of hormone levels has been shown to alter growth plate activity, longitudinal data on hormone levels in AIS patients is not typically collected, but may improve prediction accuracy [51]. Fifth, stress-scaling of growth was only performed in the vertebral bodies. Further analysis of stress scaling and unique stress sensitivity in the posterior elements may provide commentary on how significantly posterior element growth affects stress transfer through the vertebrae. Lastly, no reduction in kyphosis was observed during curve progression in the current FE model, where such reduction was observed in the patient cohort [52]. In future analysis, this unmatched trend may be addressed through either compensatory force application to represent any currently missing model scope, or through altering sagittal growth modulation characteristics.

*Utility + Future*

The modeling techniques developed in the current study modeling may provide improved insight into the prediction of scoliotic curvature progression in AIS patients. This curve progression prediction utilizing increased detail of both local growth and internal stress can be developed to assist in surgical timing, planning, and prognosis for growth-modulating interventions such as anterior vertebral body tethering (AVBT). Future models may benefit from accounting for effects of active muscle loading, overall thorax stiffness, including other soft tissue structures such as lungs (for pulmonary function assessment), and a more precise classification of deformity [53-56]. Additionally, anatomically complete FE models of progressive AIS can serve as a foundation for testing growth-modulation devices, as current human cadaveric spines or surrogates cannot mimic the growth patterns and biomechanical responses observed in children and adolescents, as well as to do so with large animal models used for device testing [24, 57, 58]. In a broader sense, predictive biomechanical models such as that developed in the current study can contribute to advancements in precision medicine and optimized clinical outcomes [59].

# References

1.	Weinstein, S.L., L.A. Dolan, J.C. Cheng, A. Danielsson, and J.A. Morcuende. Adolescent idiopathic scoliosis. Lancet. 2008. 371(9623): p. 1527-37. DOI: 10.1016/s0140-6736(08)60658-3.

2.	Yagci, G., M. Karatel, and Y. Yakut. Body Awareness and its Relation to Quality of Life in Individuals with Idiopathic Scoliosis. Perceptual and Motor Skills. 2020. 0(0) 1–17. DOI: 10.1177/0031512520924945.

3.	Stokes, I.A., D.D. Aronsson, A.N. Dimock, V. Cortright, and S. Beck. Endochondral growth in growth plates of three species at two anatomical locations modulated by mechanical compression and tension. Journal of orthopaedic research. 2006. 24(6): p. 1327-1334.

4.	Mehlman, C., A. Araghi, and D. Roy. Hyphenated history: the Hueter-Volkmann law. American journal of orthopedics (Belle Mead, N.J.). 1997. 26: p. 798-800.

5.	D'Andrea, C.R., A. Alfraihat, A. Singh, J.B. Anari, P.J. Cahill, T. Schaer, B.D. Snyder, D. Elliott, and S. Balasubramanian. Part 1. Review and meta‐analysis of studies on modulation of longitudinal bone growth and growth plate activity: A macro‐scale perspective. Journal of Orthopaedic Research. 2020(Epub ahead of print). DOI: 10.1002/jor.24976.

6.	Carrier, J., C.E. Aubin, I. Villemure, and H. Labelle. Biomechanical modelling of growth modulation following rib shortening or lengthening in adolescent idiopathic scoliosis. Med Biol Eng Comput. 2004. 42(4): p. 541-8.

7.	Driscoll, M., C.E. Aubin, A. Moreau, I. Villemure, and S. Parent. The role of spinal concave-convex biases in the progression of idiopathic scoliosis. Eur Spine J. 2009. 18(2): p. 180-7. DOI: 10.1007/s00586-008-0862-z.

8.	Huynh, A.M., C.E. Aubin, T. Rajwani, K.M. Bagnall, and I. Villemure. Pedicle growth asymmetry as a cause of adolescent idiopathic scoliosis: a biomechanical study. Eur Spine J. 2007. 16(4): p. 523-9. DOI: 10.1007/s00586-006-0235-4.

9.	Villemure, I., C.E. Aubin, J. Dansereau, and H. Labelle. Simulation of progressive deformities in adolescent idiopathic scoliosis using a biomechanical model integrating vertebral growth modulation. J Biomech Eng. 2002. 124(6): p. 784-90.

10.	Villemure, I., C.E. Aubin, J. Dansereau, and H. Labelle. Biomechanical modelling of spinal growth modulation for the study of adolescent scoliotic deformities: a feasibility study. Stud Health Technol Inform. 2002. 88: p. 373-7.

11.	Villemure, I., C.E. Aubin, J. Dansereau, and H. Labelle. Biomechanical simulations of the spine deformation process in adolescent idiopathic scoliosis from different pathogenesis hypotheses. European Spine Journal. 2004. 13(1): p. 83-90. DOI: 10.1007/s00586-003-0565-4.

12.	Kamal, Z., G. Rouhi, N. Arjmand, and S. Adeeb. A stability-based model of a growing spine with adolescent idiopathic scoliosis: A combination of musculoskeletal and finite element approaches. Medical Engineering & Physics. 2019. 64: p. 46-55. DOI: 10.1016/j.medengphy.2018.12.015.

13.	Shi, L., D. Wang, M. Driscoll, I. Villemure, W.C. Chu, J.C. Cheng, and C.E. Aubin. Biomechanical analysis and modeling of different vertebral growth patterns in adolescent idiopathic scoliosis and healthy subjects. Scoliosis. 2011. 6: p. 11. DOI: 10.1186/1748-7161-6-11.

14.	Yang, K.H. and A.I. King. Mechanism of Facet Load Transmission as a Hypothesis for Low-Back Pain. Spine. 1984. 9(6).

15.	Balasubramanian, S., C. D'Andrea, G. Viraraghavan, and P.J. Cahill. Development of a Finite Element Model of the Pediatric Thoracic and Lumbar Spine, Ribcage, and Pelvis with Orthotropic Region-Specific Vertebral Growth. Journal of Biomechanical Engineering. 2022. DOI: 10.1115/1.4054410.

16.	Peters, J.R., C. Chandrasekaran, L.F. Robinson, S.E. Servaes, R.M. Campbell, Jr., and S. Balasubramanian. Age- and gender-related changes in pediatric thoracic vertebral morphology. Spine J. 2015. 15(5): p. 1000-20. DOI: 10.1016/j.spinee.2015.01.016.

17.	Peters, J.R., S. Servaes, P. Cahill, and S. Balasubramanian. Morphology and Growth of the Pediatric Lumbar Vertebrae. The Spine Journal. 2020. DOI: 10.1016/j.spinee.2020.10.029.

18.	Peters, J.R., R.M. Campbell, and S. Balasubramanian. Characterization of the age-dependent shape of the pediatric thoracic spine and vertebrae using generalized procrustes analysis. Journal of Biomechanics. 2017. 63: p. 32-40. DOI: 10.1016/j.jbiomech.2017.07.030.

19.	Dreischarf, M., T. Zander, A. Shirazi-Adl, C.M. Puttlitz, C.J. Adam, C.S. Chen, V.K. Goel, A. Kiapour, Y.H. Kim, K.M. Labus, J.P. Little, W.M. Park, Y.H. Wang, H.J. Wilke, A. Rohlmann, and H. Schmidt. Comparison of eight published static finite element models of the intact lumbar spine: Predictive power of models improves when combined together. Journal of Biomechanics. 2014. 47(8): p. 1757-1766. DOI: https://doi.org/10.1016/j.jbiomech.2014.04.002.

20.	Qiu, T.-X. and E.-C. Teo. FINITE ELEMENT MODELING OF HUMAN THORACIC SPINE. Journal of Musculoskeletal Research. 2004. 08(04): p. 133-144. DOI: doi:10.1142/S0218957704001302.

21.	Meijer, G.J. DEVELOPMENT OF A NON-FUSION SCOLIOSIS CORRECTION DEVICE. 2011, Enschede: The Netherlands. DOI: 10.3990/1.9789036532297.

22.	Liu, X.J. and J.K. Yang. Development of Child Pedestrian Mathematical Models and Evaluation with Accident Reconstruction. Traffic Injury Prevention. 2002. 3(4): p. 321-329. DOI: 10.1080/15389580214626.

23.	Chazal, J., A. Tanguy, M. Bourges, G. Gaurel, G. Escande, M. Guillot, and G. Vanneuville. Biomechanical properties of spinal ligaments and a histological study of the supraspinal ligament in traction. J Biomech. 1985. 18(3): p. 167-76.

24.	Alfraihat, A., J.C. Olson, B.D. Snyder, P.J. Cahill, and S. Balasubramanian. Thoracic vertebral morphology in normal and scoliosis deformity in skeletally immature rabbits: A Longitudinal study. JOR SPINE. 2020. DOI: 10.1002/jsp2.1118.

25.	Peters, J.R. Geometry, Growth, and Shape of the Normative Pediatric Thoracic and Lumbar Vertebrae and Ribs, and Comparisons with Adolescent Idiopathic Scoliosis. 2017, ProQuest Dissertations Publishing.

26.	Clin, J., C.E. Aubin, N. Lalonde, S. Parent, and H. Labelle. A new method to include the gravitational forces in a finite element model of the scoliotic spine. Med Biol Eng Comput. 2011. 49(8): p. 967-77. DOI: 10.1007/s11517-011-0793-4.

27.	Mackie, E.J., Y.A. Ahmed, L. Tatarczuch, K.S. Chen, and M. Mirams. Endochondral ossification: How cartilage is converted into bone in the developing skeleton. The International Journal of Biochemistry & Cell Biology. 2008. 40(1): p. 46-62. DOI: https://doi.org/10.1016/j.biocel.2007.06.009.

28.	Lafage, V., J. Dubousset, F. Lavaste, and W. Skalli. 3D finite element simulation of Cotrel-Dubousset correction. Computer aided surgery : official journal of the International Society for Computer Aided Surgery. 2004. 9(1-2): p. 17.

29.	Fok, J., S. Adeeb, and J. Carey. FEM Simulation of Non-Progressive Growth from Asymmetric Loading and Vicious Cycle Theory: Scoliosis Study Proof of Concept. Open Biomed Eng J. 2010. 4: p. 162-9. DOI: 10.2174/1874120701004010162.

30.	Hadagali, P., J.R. Peters, and S. Balasubramanian. Morphing the feature-based multi-blocks of normative/healthy vertebral geometries to scoliosis vertebral geometries: development of personalized finite element models. COMPUTER METHODS IN BIOMECHANICS AND BIOMEDICAL ENGINEERING. 2018. 21(4): p. 297-324. DOI: 10.1080/10255842.2018.1448391.

31.	Boisvert, J., F. Cheriet, X. Pennec, H. Labelle, and N. Ayache. Articulated Spine Models for 3-D Reconstruction From Partial Radiographic Data. IEEE Transactions on Biomedical Engineering. 2008. 55(11): p. 2565-2574. DOI: 10.1109/tbme.2008.2001125.

32.	Lalonde, N.M., Y. Petit, C.E. Aubin, E. Wagnac, and P.J. Arnoux. Method to geometrically personalize a detailed finite-element model of the spine. IEEE Trans Biomed Eng. 2013. 60(7): p. 2014-21. DOI: 10.1109/tbme.2013.2246865.

33.	D'Andrea, C. Prediction of Anterior Vertebral Body Tethering Outcomes with Patient-Specific Finite Element Modelling, in Biomedical Engineering, Science, and Health Systems. 2020, Drexel University.

34.	Viraraghavan, G. Development of Deformity Specific Finite Element Models for Surgical Simulation of Anterior Vertebral Body Tether for Treating Scoliosis in Pediatric Subjects. 2019, Drexel University.

35.	Dong, L., G. Li, H. Mao, S. Marek, and K.H. Yang. Development and validation of a 10-year-old child ligamentous cervical spine finite element model. Annals of biomedical engineering. 2013. 41(12): p. 2538-2552. DOI: 10.1007/s10439-013-0858-7.

36.	El Masri, F., E. Sapin de Brosses, K. Rhissassi, W. Skalli, and D. Mitton. Apparent Young's modulus of vertebral cortico-cancellous bone specimens. Comput Methods Biomech Biomed Engin. 2012. 15(1): p. 23-8. DOI: 10.1080/10255842.2011.565751.

37.	Forman, J.L. and R.W. Kent. Modeling costal cartilage using local material properties with consideration for gross heterogeneities. Journal of Biomechanics. 2011. 44(5): p. 910-916. DOI: 10.1016/j.jbiomech.2010.11.034.

38.	Feng, J., T. Hu, W. Liu, S. Zhang, Y. Tang, R. Chen, X. Jiang, and F. Wei. The biomechanical, morphologic, and histochemical properties of the costal cartilages in children with pectus excavatum. Journal of Pediatric Surgery. 2001. 36(12): p. 1770-1776. DOI: https://doi.org/10.1053/jpsu.2001.28820.

39.	Yoganandan, N. and F.A. Pintar. Biomechanics of Human Thoracic Ribs. Journal of Biomechanical Engineering. 1998. 120(1): p. 100-104. DOI: 10.1115/1.2834288.

40.	Aira, J., B. Guleyupoglu, D. Jones, B. Koya, M. Davis, and F.S. Gayzik. Validated thoracic vertebrae and costovertebral joints increase biofidelity of a human body model in hub impacts. Traffic Injury Prevention. 2019. 20(sup2): p. S1-S6. DOI: 10.1080/15389588.2019.1638511.

41.	Cobetto, N., S. Parent, and C.E. Aubin. 3D correction over 2years with anterior vertebral body growth modulation: A finite element analysis of screw positioning, cable tensioning and postoperative functional activities. Clin Biomech (Bristol, Avon). 2018b. 51: p. 26-33. DOI: 10.1016/j.clinbiomech.2017.11.007.

42.	Modi, H.N., C.H. Modi, S. Suh, J.-H. Yang, and J.-Y. Hong. Correlation and comparison of Risser sign versus bone age determination (TW3) between children with and without scoliosis in Korean population. Journal of Orthopaedic Surgery and Research. 2009. 4(1): p. 36. DOI: 10.1186/1749-799x-4-36.

43.	Morrissy, R.T., G.S. Goldsmith, E.C. Hall, D. Kehl, and G.H. Cowie. Measurement of the Cobb angle on radiographs of patients who have scoliosis. Evaluation of intrinsic error. The Journal of bone and joint surgery. American volume. 1990. 72(3): p. 320-327.

44.	Chuah, S.L., B.A. Kareem, K. Selvakumar, K.S. Oh, A. Borhan Tan, and S. Harwant. The natural history of scoliosis: curve progression of untreated curves of different aetiology, with early (mean 2 year) follow up in surgically treated curves. Med J Malaysia. 2001. 56 Suppl C: p. 37-40.

45.	Welborn, M.C., R. Coghlan, S. Sienko, and W. Horton. Correlation of collagen X biomarker (CXM) with peak height velocity and radiographic measures of growth in idiopathic scoliosis. Spine Deformity. 2021. 9(3): p. 645-653. DOI: 10.1007/s43390-020-00262-7.

46.	Sanders, J.O., J.G. Khoury, S. Kishan, R.H. Browne, J.F. Mooney, III, K.D. Arnold, S.J. McConnell, J.A. Bauman, and D.N. Finegold. Predicting Scoliosis Progression from Skeletal Maturity: A Simplified Classification During Adolescence. JBJS. 2008. 90(3).

47.	Drevelle, X., Y. Lafon, E. Ebermeyer, I. Courtois, J. Dubousset, and W. Skalli. Analysis of idiopathic scoliosis progression by using numerical simulation. Spine (Phila Pa 1976). 2010. 35(10): p. E407-12. DOI: 10.1097/BRS.0b013e3181cb46d6.

48.	Negrini, S., C. Fusco, S. Minozzi, S. Atanasio, F. Zaina, and M. Romano. Exercises reduce the progression rate of adolescent idiopathic scoliosis: Results of a comprehensive systematic review of the literature. Disability and Rehabilitation. 2008. 30(10): p. 772-785. DOI: 10.1080/09638280801889568.

49.	Weinstein, S.L., L.A. Dolan, J.G. Wright, and M.B. Dobbs. Effects of Bracing in Adolescents with Idiopathic Scoliosis. New England Journal of Medicine. 2013. 369(16): p. 1512-1521. DOI: 10.1056/nejmoa1307337.

50.	Cheng, J.C., R.M. Castelein, W.C. Chu, A.J. Danielsson, M.B. Dobbs, T.B. Grivas, C.A. Gurnett, K.D. Luk, A. Moreau, P.O. Newton, I.A. Stokes, S.L. Weinstein, and R.G. Burwell. Adolescent idiopathic scoliosis. Nature Reviews Disease Primers. 2015. 1(1): p. 15030. DOI: 10.1038/nrdp.2015.30.

51.	Karimian, E., A.S. Chagin, and L. Sävendahl. Genetic Regulation of the Growth Plate. Frontiers in Endocrinology. 2012. 2. DOI: 10.3389/fendo.2011.00113.

52.	Zhang, C., Y. Wang, J. Yu, F. Jin, Y. Zhang, Y. Zhao, Y. Fu, K. Zhang, J. Wang, L. Dai, M. Gao, Z. Li, L. Wang, X. Li, and H. Wang. Analysis of sagittal curvature and its influencing factors in adolescent idiopathic scoliosis. Medicine. 2021. 100(23).

53.	Orbach, M.R., S.E. Servaes, O.H. Mayer, P.J. Cahill, and S.J.P.P. Balasubramanian. Quantifying lung and diaphragm morphology using radiographs in normative pediatric subjects, and predicting CT‐derived lung volume. 2021. 56(7): p. 2177-2185.

54.	Mathews, E.A., S. Balasubramanian, T. Seacrist, M.R. Maltese, R. Sterner, K.B.J.J.o.e. Arbogast, and kinesiology. Electromyography responses of pediatric and young adult volunteers in low-speed frontal impacts. 2013. 23(5): p. 1206-1214.

55.	Arbogast, K.B., A. Nishisaki, S. Balasubramanian, J. Nysaether, D. Niles, R.M. Sutton, K.E. Roberts, L. Nadkarni, J. Boulet, and M.R.J.R. Maltese. Expert clinical assessment of thorax stiffness of infants and children during chest compressions. 2009. 80(10): p. 1187-1191.

56.	Harris, J.A., O.H. Mayer, S.A. Shah, R.M. Campbell, and S.J.E.S.J. Balasubramanian. A comprehensive review of thoracic deformity parameters in scoliosis. 2014. 23(12): p. 2594-2602.

57.	Bianco, R.-J., P.-J. Arnoux, E. Wagnac, J.-M. Mac-Thiong, and C.-É. Aubin. Minimizing Pedicle Screw Pullout Risks. Clinical Spine Surgery. 2017. 30(3): p. E226-E232. DOI: 10.1097/bsd.0000000000000151.

58.	Balasubramanian, S., J.R. Peters, L.F. Robinson, A. Singh, and R.W. Kent. Thoracic spine morphology of a pseudo-biped animal model (kangaroo) and comparisons with human and quadruped animals. 2016. 25(12): p. 4140-4154.

59.	Mandel, W., O. Turcot, D. Knez, S. Parent, and S. Kadoury. Prediction outcomes for anterior vertebral body growth modulation surgery from discriminant spatiotemporal manifolds. International Journal of Computer Assisted Radiology and Surgery. 2019. DOI: 10.1007/s11548-019-02041-w.